ALU CONTROLLER

PROJECT DOCUMENTATION

ALU  DESIGN

Logisim. The three memories contain the test inputs. You can use the poke tool  to set the Test # register

to the Test # given in the error message. Our test’s error message said

Test #  ALUFlags    Result ...

1       8           87a08d79

1       9           87a08d79

The Test # is 1, so we would want to set the Test # register to 1 to see which inputs for A, B, and ALUControl were being tested.

In this case it was A=0x23409824, B=0x645ff555, ALUControl=7 (that is, add). Indeed, this addition should lead to an overflow, but our ALU is outputting that overflow=0.

From here, to see why our ALU is outputting the wrong answer, right click the ALU and choose view Main.

This will take you into your alu.circ. The test inputs will be coming in so that you can investigate what is happening on all the wires. Trace wires backwards from output towards the input. That is, start at the ALUFlags output, and look for where oVerflow is coming from. Keep tracing backwards until you find an unexpected value on a wire; that will help you find the cause.

• Tip: To see the value on a wire either:

o Use the poke tool (hand with pointer finger) o Or place a Probe, found under Wiring folder

The starter code contains the following files

- Makefile: contains commands for running the tests

- alu.circ: the skeleton file where you should implement your ALU

- alu-control: the skeleton file where you should implement your ALU Control

- tests/ o alu-add.circ: tests alu.circ’s add operation o alu-asr.circ: tests alu.circ’s arithmetic shift right operation o alu-control-b-test.circ: tests alu-control.circ for b instruction

  - alu-control-basic-data-processing-test.circ: tests alu-control.circ for basic dataprocessing instructions

  - alu-control-cmp-test.circ: tests alu-control.circ for cmp instruction o alu-control-ldr-str-test.circ: tests alu-control.circ for ldr and str instructions o alu-control-shifts-test.circ: tests alu-control.circ for shift instructions o test.py: python script for testing your circuits; this is also where the expected outputs of the tests go o decode_out.py: python script to format Logisim output

  - logisim.jar: a copy of Logisim used by the test code.

The ALU is a component that performs one of a number of arithmetic operations on two 32-bit inputs.

## Description

The inputs and outputs of the ALU are as follows.

Input

| Name | Bit width | Description |
| --- | --- | --- |
| A | 32 | First operand |
| B | 32 | Second operand |
| ALUControl | 4 | the operation the ALU should compute to obtain the value of output Result |

Output

| Name | Bit width | Description |
| --- | --- | --- |
| ALUFlags | 4 | {Negative, Zero, Carry, oVerflow} |
| Result | 32 | Result of the operation |

Here are the individual bits in the ALUFlags

| Name | Bit width | Description |
| --- | --- | --- |
| oVerflow | 1 | 1 iff operation is add or sub and there was signed overflow |
| Carry | 1 | 1 iff operation is add or sub and there was carry out |
| Zero | 1 | 1 iff Result is equal to 0 |
| Negative | 1 | 1 iff Result is negative |

Here is the specification for the Result.

| ALUControl | ALU operation name (NOT an ARM instruction) | Result |
| --- | --- | --- |
| 0 | and | A AND B |
| 1 | or | A OR B |
| 2 | xor | A XOR B |
| 3 | not | NOT A |
| 4 | lsl | B << A |
| 5 | lsr | B >> A (logical) |
| 6 | asr | B >> A (arithmetic) |
| 7 | add | A+B |
| 8 | sub | A-B |

RESULTS

v2.0 raw

20010001

00210820

00210820

00210820

00210820

00210820

AC010000

8C020000

00211824

AC630000

8C640000

AC64000